BMW E30 3 Series Specifications

By Jared Fenton

## Engine Specifications:

| Model | 318i 1984-85 | 318i/is/iC 1990-92 | 325/e/es 1984-87 | 325i/is/iC/iX 1987-92 | M3 1987-91 |
| --- | --- | --- | --- | --- | --- |
| Engine Code | M10 | M42B18 | M20B27 | M20B25 | S14 |
| Cylinders | 4 | 4 | 6 | 6 | 4 |
| Bore mm (in,) | 89.0 (3.504) |  | 89.0 (3.504) | 89.0 (3.504) | 93.4 (3.679) |
| Stroke mm (in,) | 71.0 (2.795) |  | 81.0 (3.189 | 75.0 (2.953) | 84.0 (3.307) |
| Displacement cc (cu. In.) | 1766 (107.8) | 1796 (109.5) | 2693 (164.0) | 2492 (152.0) | 2305 (140.6) |
| Compression Ratio | 9.0:1 | 10.0:1 | 9.0:1 | 8.8:1 | 10.5:1 |
| Horsepower SAE net @rpm | 101 @ 5800 | 136 @ 6000 | 121 @ 4250 | 168 @ 5800 | 192 @ 6750 |
| Torque lbs.-ft. @rpm SAE net | 103 @ 4500 | 127 @ 4600 | 170 @ 3250 | 164 @ 4300 | 170 @ 4750 |
| Fuel Delivery | Bosch L-Jetronic | Bosch Motronic | Bosch Motronic | Bosch Motronic 1.1 | Bosch Motronic |
| Firing Order | 1-3-4-2 | 1-3-4-2 | 1-5-3-6-2-4 | 1-5-3-6-2-4 | 1-3-4-2 * |

Transmission Specifications:

*    Later 4 Cylinder cars equipped with either 4HP 22 or 4HP 22EH

**  Early 4 Cylinder cars were also equipped with the ZF S5-16 Transmission, check driver’s side of transmission casing for identification tag.

| Model |  | 318i 1984-85 |  |  | 318is/iC 1990-92 |  | 325/e/es 1984-87 |  | 325i/is/iC/iX 1987-92 |  | M3 1987-91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Automatic Transmission Type |  | ZF 3 HP 22* |  |  | N/A |  | 4HP 22 or 4HP 22EH |  | 4HP 22 or 4HP 22EH |  | N/A |
| Manual Transmission Type |  | Getrag 240** |  |  | Getrag 260/5 |  | Getrag 260 |  | Getrag 260 |  | Getrag 260/5 |
|  | Manual Transmission Fluid |  | 1.2 qt. (1.15 L) | 1.2 qt. (1.1 L) |  | 1.32 qt. (1.25 L) |  | 1.32 qt. (1.25 L) |  | 1.6 qt. (1.51 L) |  |
|  | Automatic Transmission Fluid |  | 2.1 Qt. (2.0 L) | 3.2 qt. (3.0 L) |  | 3.2 qt. (3.0 L) |  | 3.2 qt. (3.0 L) |  | N/A |  |

Manual Transmission Gear Ratios:

| Model | Getrag 240 | Getrag 260 | ZF S5-16 | Getrag 260/5 |
| --- | --- | --- | --- | --- |
| 1st Gear | 3.72 | 3.83 | 3.72 | 3.72 |
| 2nd Gear | 2.02 | 2.04 | 2.2 | 2.02 |
| 3rd Gear | 1.32 | 1.34 | 1.4 | 1.32 |
| 4th Gear | 1 | 1 | 1 | 1 |
| 5th Gear | 0.81 | 0.8 | 0.81 | 0.8 |
| Reverse Gear | 3.45 | 3.54 | 3.46 | 3.46 |

Automatic Transmission Gear Ratios:

| Model | ZF 3HP 22 | ZF 4HP 22 | ZF 4HP 22EH |
| --- | --- | --- | --- |
| 1st Gear | 2.73 | 2.48 | 2.48 |
| 2nd Gear | 1.56 | 1.48 | 1.48 |
| 3rd Gear | 1 | 1 | 1 |
| 4th Gear | N/A | 0.73 | 0.73 |
| Reverse Gear | 2.09 | 2.09 | 2.09 |

Final Drive/ Differential Specifications:

Please keep in mind that several different ratios were available with these cars. These ratios are the stock specifications.

| Model | 318i 1984-85 | 318i/is/iC 1990-92 | 325/e/es 1984-87 | 325i/is/iC/iX 1987-92 | M3 1987-91 |
| --- | --- | --- | --- | --- | --- |
| Final Drive (Auto Trans.) | 3.91:1 | N/A | 2.78:1 | 2.91:1 | N/A |
| Final Drive (Manual Trans.) | 3.91:1 | 4.10:1 | 2.78:1 | 3.73:1 | 4.10:1 |
| Differential Fluid | 0.95 qt. (0.9 L) | 1.2 qt. (1.1 L) | 1.9 qt. (1.7 L) | 1.9 qt. (1.7 L) | 1.9 qt. (1.7 L) |

## Fluids and Lubrication Capacities:

* Indicates oil amount with filter changed.

| Model | 318i 1984-85 | 318i/is/iC 1990-92 | 325/e/es 1984-87 | 325i/is/iC/iX 1987-92 | M3 1987-91 |
| --- | --- | --- | --- | --- | --- |
| Engine Oil | 4.26 qt. (3.75 L)* | 5.3 qt. (5.0 L)* | 4.46 qt. (4.25 L)* | 4.46 qt. (4.25 L)* |  |
| Differential Fluid | 0.95 qt. (0.9 L) | 1.2 qt. (1.1 L) | 1.9 qt. (1.7 L) | 1.9 qt. (1.7 L) | 1.9 qt. (1.7 L) |
| Brake Fluid | Top up as Needed | Top up as Needed | Top up as Needed | Top up as Needed | Top up as Needed |
| Power Steering Fluid | Top up as Needed | Top up as Needed | Top up as Needed | Top up as Needed | Top up as Needed |
| Engine Coolant | 7.4 qt. (7.0 L) | 6.9 qt. (6.5 L) | 12.7 qt. (12.0 L) | 11.0 qt. (10.5 L) | N/A |

Spark Plug Specifications:

* Gap may have a +/- tolerance of 0.1mm (0.004 in.)

| Model; 318i 1984-85; 318i/is/iC 1990-92; 325/e/es 1984-87; 325 1988; 325i/is/iC/iX 1987-92. Spark Plug (Bosch); WR9DS; F8 LCR; WR9LS; WR9LS; W8LCR. Spark Plug (NGK); BPR 5 EIX; BKR 6 EK; ZGR 5 A; ZGR 5 A; N/A. Electrode Gap; 0.8mm (0.032 in.)*; 0.8mm (0.032 in.)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*. Tightening Torque; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.. | Model; 318i 1984-85; 318i/is/iC 1990-92; 325/e/es 1984-87; 325 1988; 325i/is/iC/iX 1987-92. Spark Plug (Bosch); WR9DS; WR9LS; WR9LS; W8LCR. Spark Plug (Beru); RS 33; 14 R/9 LS; 14 R/9 LS; N/A. Electrode Gap; 0.8mm (0.032 in.)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*. Tightening Torque; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.. | Model; 318i 1984-85; 318i/is/iC 1990-92; 325/e/es 1984-87; 325 1988; 325i/is/iC/iX 1987-92. Spark Plug (Bosch); WR9DS; WR9LS; WR9LS; W8LCR. Spark Plug (Beru); RS 33; 14 R/9 LS; 14 R/9 LS; N/A. Electrode Gap; 0.8mm (0.032 in.)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*. Tightening Torque; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.. | Model; 318i 1984-85; 318i/is/iC 1990-92; 325/e/es 1984-87; 325 1988; 325i/is/iC/iX 1987-92. Spark Plug (Bosch); WR9DS; WR9LS; WR9LS; W8LCR. Spark Plug (Beru); RS 33; 14 R/9 LS; 14 R/9 LS; N/A. Electrode Gap; 0.8mm (0.032 in.)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*; 0.7mm (0.027 in)*. Tightening Torque; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.; 17 ft./lbs.. |
| --- | --- | --- | --- |

Valve Clearance Specifications:

*    1/2 turn between cylinders (approx. 180 degrees)

**  1/3 turn between cylinders (approx. 120 degrees)

| Model | 318i 1984-85 | 318i/is/iC 1990-92 | 325/e/es 1984-87 | 325i/is/iC/iX 1987-92 | M3 1987-91 |
| --- | --- | --- | --- | --- | --- |
| Valve Clearance (warm) | 0.25mm (0.010in.) | Non-adjustable | 0.30mm (0.012in.) | 0.30mm (0.012in.) | 0.30mm (0.012in.) |
| Valve Clearance (cold) | 0.20mm (0.008in.) | Non-adjustable | 0.25mm (0.010in.) | 0.25mm (0.010in.) | 0.25mm (0.010in.) |
| Valve Adjustment Sequence | 1-3-4-2 * | N/A | 1-5-3-6-2-4 ** | 1-5-3-6-2-4 ** | 1-3-4-2 * |

Vehicle Dimensions:

| Model | 318i 1984-85 | 318i/is/iC 1990-92 | 325/e/es 1984-87 | 325i/is/iC/iX 1987-92 | M3 1987-91 |
| --- | --- | --- | --- | --- | --- |
| Curb Weight (Manual Trans.) | 2360 lbs. | 2602 lbs | 2654 lbs. | 2813 lbs. | 2865 lbs. |
| Curb Weight (Auto Trans.) | 2380 lbs. | N/A | 2698 lbs. | 2865 lbs. | N/A |
| Curb Weight (Convertible) | N/A | 2867 lbs. | N/A | 3015 lbs. | N/A |
| Curb Weight (4 Door) | 2450 lbs. | 2657 lbs. | 2765 lbs. | 2895 lbs. | N/A |
| Wheelbase | 101.2 in. | 101.2 in. | 101.2 in. | 101.2 in. | 100.9 in. |
| Weight Distribution front/rear | 53/47 | 54/46 | 53/47 | 53/47 | 53/47 |
| Fuel Capacity | 14.5 gal. (54.88 L) | 14.5 gal. (54.88 L) | 14.5 gal. (54.88 L) | 14.5 gal. (54.88 L) | 14.5 gal. (54.88 L) |